S.SOUBAN

192312181

Slot: A

Channel Reuse Plan for Multi-User Wireless Communication

Concept: To design and simulate a channel reuse plan for a multi-user wireless communication system by dividing the coverage area into cells and assigning available channels to different cells using a suitable frequency reuse factor, while minimizing co-channel interference.

Matlab Cobe:

clc;

clear;

close all;

% Number of cells

N = 7;

% Total number of available channels

total_channels = 21;

% Channels per cell

channels_per_cell = total_channels / N;

% Frequency reuse factor

reuse_factor = N;

% Cell radius (km)

R = 1;

% Create approximate 7-cell layout

x = [0 1.5 0.75 -0.75 -1.5 -0.75 0.75];

y = [0 0.87 1.73 1.73 0.87 0 -0.87];

% Channel allocation

channel_groups = [

1  2  3;

4  5  6;

7  8  9;

10 11 12;

13 14 15;

16 17 18;

19 20 21

];

% Display channel allocation

disp('Channel Allocation:');

for i = 1:N

fprintf('Cell %d: Channels %d, %d, %d\n', ...

i, channel_groups(i,1), ...

channel_groups(i,2), ...

channel_groups(i,3));

end

% Plot cells

figure;

hold on;

grid on;

axis equal;

theta = linspace(0,2*pi,7);

for i = 1:N

% Hexagonal cell

hx = x(i) + R*cos(theta);

hy = y(i) + R*sin(theta);

fill(hx, hy, 'w');

plot(hx, hy, 'k', 'LineWidth', 1.5);

% Cell number

text(x(i), y(i), sprintf('Cell %d', i), ...

'HorizontalAlignment', 'center', ...

'FontWeight', 'bold');

% Channel numbers

text(x(i), y(i)-0.25, ...

sprintf('Ch: %d,%d,%d', ...

channel_groups(i,1), ...

channel_groups(i,2), ...

channel_groups(i,3)), ...

'HorizontalAlignment', 'center');

end

title('7-Cell Frequency Reuse Plan');

xlabel('Distance (km)');

ylabel('Distance (km)');

hold off;

% Calculate theoretical co-channel reuse distance

D = sqrt(3*N) * R;

fprintf('\nFrequency Reuse Factor = %d\n', reuse_factor);

fprintf('Channels per Cell = %.0f\n', channels_per_cell);

fprintf('Cell Radius = %.2f km\n', R);

fprintf('Co-channel Reuse Distance = %.2f km\n', D);

Output:

Result: The 7-cell channel reuse plan was successfully simulated in MATLAB. The 21 available channels were divided into 7 cell groups, with 3 channels assigned to each cell. The frequency reuse factor was found to be , and the calculated co-channel reuse distance was approximately 4.58 km for a cell radius of 1 km. This arrangement improves spectrum utilization while reducing co-channel interference.

Smart City Communication Framework for Traffic Monitoring and Surveillance

Concept: To design and simulate a smart city communication framework for real-time traffic monitoring and surveillance using cameras, sensors, wireless communication networks, edge/cloud processing, and a central traffic management system.

Matlab Code:

clc;

clear;

close all;

% Smart City Traffic Monitoring Simulation

% Number of traffic locations

num_locations = 6;

% Number of vehicles detected at each location

vehicles = [35 52 78 45 90 63];

% Average vehicle speed (km/h)

speed = [45 38 25 42 20 32];

% Traffic density levels

density = vehicles / max(vehicles) * 100;

% Communication latency in milliseconds

latency = [25 30 42 28 50 35];

% Packet delivery ratio (%)

PDR = [98 97 95 99 93 96];

% Display results

fprintf('SMART CITY TRAFFIC MONITORING SYSTEM\n');

fprintf('-------------------------------------\n');

for i = 1:num_locations

fprintf(['Location %d: Vehicles = %d, Speed = %d km/h, ' ...

'Density = %.1f%%, Latency = %d ms, PDR = %.1f%%\n'], ...

i, vehicles(i), speed(i), density(i), ...

latency(i), PDR(i));

end

% Identify congested locations

threshold = 70;

fprintf('\nTraffic Status:\n');

for i = 1:num_locations

if density(i) >= threshold

fprintf('Location %d: HIGH TRAFFIC - ALERT GENERATED\n', i);

elseif density(i) >= 40

fprintf('Location %d: MEDIUM TRAFFIC\n', i);

else

fprintf('Location %d: LOW TRAFFIC\n', i);

end

end

% Plot traffic density

figure;

bar(1:num_locations, density);

title('Traffic Density at Different Locations');

xlabel('Traffic Monitoring Location');

ylabel('Traffic Density (%)');

grid on;

% Plot communication latency

figure;

plot(1:num_locations, latency, '-o', 'LineWidth', 2);

title('Communication Latency');

xlabel('Traffic Monitoring Location');

ylabel('Latency (ms)');

grid on;

% Plot packet delivery ratio

figure;

bar(1:num_locations, PDR);

title('Packet Delivery Ratio');

xlabel('Traffic Monitoring Location');

ylabel('PDR (%)');

ylim([85 100]);

grid on;

% Overall average performance

avg_latency = mean(latency);

avg_PDR = mean(PDR);

avg_density = mean(density);

fprintf('\nOverall Performance:\n');

fprintf('Average Traffic Density = %.2f%%\n', avg_density);

fprintf('Average Communication Latency = %.2f ms\n', avg_latency);

fprintf('Average Packet Delivery Ratio = %.2f%%\n', avg_PDR);

OutPut:

Result: The smart city communication framework for traffic monitoring and surveillance was successfully designed and simulated in MATLAB. The system monitors vehicle density, vehicle speed, communication latency, and packet delivery ratio at multiple locations. Locations with high traffic density are automatically identified and an alert is generated. The framework demonstrates how wireless communication, edge processing, and centralized traffic management can support real-time traffic monitoring and congestion detection.